Wipro Service Delivery

Standard of Procedure

for

Port Provisioning

Network-Data/03-Aug-17/Process

Table of contents

1.	Purpose	3

2.	Business as Usual (BAU) Change	3

2.1 Port Provisioning	3

3.	Roles & Responsibility	3

4.	Environment	3

5.	Definitions & Acronyms	3

6.	References	3

7.	Implementation Steps	4

8.	Amendment History	7

# Purpose

The Purpose of this Steps of Procedure (SOP) is to:

- Identify the steps for implementation of BAU Changes.

# Business as Usual (BAU) Change

- 2.1 Port Provisioning

The following steps provide a high-level overview that outlines Port Provisioning updates from request to completion.

Port Provisioning:

- A service request is opened with the YYZ Service Desk for Port Provisioning

- Request should be assigned to Service Provisioning (Black & McDonald) to provide the interface(s) details for the cable/Data Jack id of the port to be provisioned by network team,

- if switch, port and vlan details already available in the ticket description it will be assigned to Network-Data team.

- New ports that are being provisioned and do not have any devices connected to them have no impact as they are new services being provisioned.

- If requested vlan is available on the switch Port provisioning configurations implemented.

- If requested vlan is not available on the switch Network Data team will create a BAU change request (RFC) to provision the interface(s) as per provided information.

- The service request ticket is updated with the change request (RFC) number.

- RFC approved and executed as per date/time indicated in the change.

- Port provisioning configurations implemented and reviewed to ensure they match the requested requirements. (See below for port provisioning steps)

- Service request is updated with status and closed.

# Roles & Responsibility

Network Team – Implementing the changes in network

# Environment

environment is comprised of Cisco equipment’s.

# Definitions & Acronyms

# References

NA

# Implementation Steps

- Gather data provided in the ticket like Switch, port, Vlan details, either given by the user directly, or by vendor B&M/Servicetec which needs to be provisioned.

- Look for the Vlan which needs to be provisioned in that port, which would be provided by the user/tenant.

- RDP(mstsc) to Wipro Jump box 10.251.208.11/10.251.208.10

- RDP(mstsc) to Network Jump box 192.168.3.142/192.168.3.24

- Login to the provided switch (mentioned in the ticket) on which the port needs to be provisioned via Putty/SecureCRT.

- Follow execution with respect to scenarios applicable as below.

- Show vlan id <vlan number> and check if the vlan is present on that switch

- Check if the Vlan is allowed on the uplink trunk port of the access switch

- If yes

- For Data Vlan:

- execute following commands:

#Conf t

#Interface <number>

#Switchport

#Switchport mode access

#Switchport access vlan <vlan number>

#spanning-tree portfast edge

#switch-port security

# Speed <auto/10/100/1000>

# description <Cable id_System/Tenant name_SR Number>

# exit

# exit

- To verify the provisioning:

#Show interface <number> status

- For Data and Voice Vlan:

#Conf t

#Interface <number>

#Switchport

#Switchport mode access

#Switchport access vlan <vlan number>

#Switchport voice vlan <vlan number>

#spanning-tree portfast edge

#switch-port security

# switch-port security maximum 2

# Speed <auto/10/100/1000>

# description <Cable id_System/Tenant name_SR Number>

# exit

# exit

- To verify the provisioning:

#Show interface <Number> status

#Show mac address-table interface (number)

- For Standalone Voice Vlan Provisioning:

- For Voice Vlan:

#Conf t

#Interface <number>

#Switchport

#Switchport mode access

#Switchport access vlan 2

#Switchport voice vlan <vlan number>

#spanning-tree portfast edge

#switch-port security

# switch-port security maximum 2

# Speed <auto/10/100/1000>

# description <Cable id_System/Tenant name_SR Number>

# exit

# exit

- To verify the provisioning:

#Show interface status

#Show mac address-table interface <number>

- If No:

- If Vlan is not available refer 2.1[4] for process.

- Check for Vlan in Access Switch, and the uplinks connected to Distribution Switch using:

- show cdp neighbor | sec 200.

- Check for the uplinks and check the configuration on those uplinks and check if the Vlan is passed or not?

- If the Vlan is not trunked, follow the below steps:

- In the Access Switch, creation of Vlan:

AccessSwitch#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

AccessSwitch(config)#vlan <Id_No>

AccessSwitch(config-vlan)#name <Vlan_name>

AccessSwitch(config-vlan)#exit

- Check if the Vlan got created by using the following command

AccessSwitch#sh vlan id <Id_No>

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

Id_No. Vlan_name                active    Gi5/1, Gi6/1, Gi7/40

- In distribution switch the Vlan should be passed on the thse uplink connected to Access Switch, for e.g. Gi1/13 below, and allow Vlan to be trunked on that uplink by following the below steps:

Distribution_1#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Distribution_1(config)#int Gi1/13

Distribution_1(config-if)#switchport trunk allowed vlan add <Id_No>

Distribution_1(config-if)#exit

Distribution_1(config)#exit

- Check if the Vlan got trunked on the given port by using the following steps

Distribution_1#sh vlan id <Id_No>

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

Id_No. Vlan_name                active    Gi1/4, Gi1/9, Gi1/13, Gi1/24, Po1

- In second distribution switch the Vlan should be passed on the the uplink connected to Access Switch, for e.g. Gi1/13 below, and allow Vlan to be trunked on that uplink by following the below steps:

Distribution_2#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

Distribution_2(config)#interface Gi1/13

Distribution_2(config-if)#switchport trunk allowed vlan add Id_No.

Distribution_2(config-if)#exit

Distribution_2(config)#exit

- Check if the Vlan got trunked on the given port by using the following steps

Distribution_2#sh vlan id <Id_No>

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

Id_No. Vlan_name                active    Gi1/4, Gi1/9, Gi1/13, Gi1/24, Po1

# Amendment History

| Version | Author(s) | Approved By | Nature of Change |
| --- | --- | --- | --- |
| 0.1 | Divya Kumari |  | Creation |
| 0.2 | Kiran Patel |  | Template Update 03/08/17 |
| 1.0 | Yogesh Rajguru |  | Reviewed & approved |

# Thank You